ENGR 12     				PRACTICE TEST  1				SOLUTIONS

These are representative problems. The actual test will be shorter, but you should expect a number of short drill type questions with 1 or two extended problems similar to the material covered in these examples.

| - The power associated with the 8A Current Source is - 256 W, producing - 704 W, absorbing - 576 W, producing - 384 W, absorbing |  |
| --- | --- |

- For the following circuit, the voltage drop across the 12 Ohm Resistor in Volts is:

  - 2.57142      b.    4.66666       c. 5.45454      d.    5.76

- For the following circuit, it has been determined that  ia = 1 Amp and ib  = 0.5 Amps.  Using KVL, OL and KCL, find           vc,  ic  and then id.          Ignore all other components except those that are labeled.

| The value of id in Amps is - 0.06 - 0.3 - — 0.1 - 0.22 Vc = 10V Ic = .4A Id = -0.1A |  |
| --- | --- |

| - If the interconnections of ideal sources are valid, - Identify which sources are developing and which sources are releasing power - Verify that the total power developed in the circuit equals the total power absorbed 60V: P=480 Absorbing 20V: P=160 Producing 8A: P = 320 Producing |  |
| --- | --- |

- For the following circuit

  - Find Rab for the resistive network shown (HINT: it’s a whole number, no decimal)

  - Find the total power delivered by the voltage source

  - Find the voltage drop across the 20 Ohm Resistor

TRICK!   13 || 0 (short) = 0, so remove 13 Ohm resistor
a) Rab = 7 + 20||( 6||3  + 3) = 7 + 20||5 = 11

b) Itotal = 22/11 = 2A,  P = 2*22 = 44W

c) Reff = 20||5 = 4 Ohms,   so V20 = 4*22/(11) = 8V

- For the following circuit, it has been determined that  ia = .5 Amps.  Using KCL, KVL and OL,

| - Find vb = 16 - Find ib = 0.5 - Find ic = 1 - Find vd = 28 - Find ie = 3 - The power delivered by the 40V source = 120W |  |
| --- | --- |

Hint: Answers to d, e, and f should be whole numbers