ENGR 12     			TEST  3	      100 Points				NAME: ________________

| - (10 pts) A 0.1H inductor is driven by the a current pulse i(t) = 4t-t2 Amps over the time interval 0<=t<=4 seconds. The current is 0 outside of this time period. |  |
| --- | --- |

  - Find the voltage over this same time period:

  - What is the energy in the inductor at   0 sec:________________________________

2 sec__________________________________

- (20 pts) Find the steady state solution for io(t) if vg(t) = 40cos(500t) Volts

- (30 pts) The switch has been open in the following circuit for a long time before closing at t=0.

Find

- this is a (circle one)  natural/step    response

- the initial current i  in the inductor at t=0-  ______________

- the final current  i  in the inductor at t = infinity ______________

- the effective resistance seen by the inductor for t>0 ___________________

- time constant tau for t>0  _____________________________

- the expression for i(t) for t>0  _________________________

- (40 pts) The switch has been closed a long time before opening at t=0

- this is a  (circle one of each pair)    series/parallel        step/natural    response

- Find IL(0-) ______________________

- Find Vc(0-)  _____________________

- Find IL(∞) ______________________

- Find Vc(∞) ______________________

- Find   ________________________

- Find wo ________________________

- Critically Damped, Underdamped or Overdamped ? ______________________________

- Find  dVc/dt(0+) ___________________________

- Find the complete solution for v(t) for t>0