ENGR 12     			TEST  3	      100 Points				NAME: ________________

| - (10 pts) A 0.1H inductor is driven by the a current pulse i(t) = 4t-t2 Amps over the time interval 0<=t<=4 seconds. The current is 0 outside of this time period. |  |
| --- | --- |

  - Find the voltage over this same time period:

.1( 4 – 2t)  V

  - What is the energy in the inductor at   0 sec:____0____________________________

2 sec______________0.8J____________________

- (20 pts) Find the steady state solution for io(t) if vg(t) = 40cos(500t) Volts

W = 500

ZL = j 3.2*500= 1600j

ZC = -j/(500*2.5x10^-6)  = -800 j

Ztotal = 600 + j(1600-800)    =  600 + 800 j

Vg = 40<0

Io = Vg/Ztotal = 40 / (600 +800j)  =  .024 - .032i   = .04 < -53.13

Io(t) = 40 cos (500t -53.13 )   mA

- (30 pts) The switch has been open in the following circuit for a long time before closing at t=0.

Find

- this is a (circle one)  natural/step    response

- the initial current i  in the inductor at t=0-  __125/50  = 2.5A___

- the final current  i  in the inductor at t = infinity __0A___

- the effective resistance seen by the inductor for t>0 __25 Ohm_________________

- time constant tau for t>0  ______L/R  .05/25 = .002s

- the expression for i(t) for t>0  ______2.5 e –t/.002_=__2.5 e –500t

- (40 pts) The switch has been closed a long time before opening at t=0

- this is a  (circle one of each pair)    series/parallel        step/natural    response

- Find IL(0-) ______72/4800 = 15mA_______

- Find Vc(0-)  _____0____

- Find IL(∞) _____0_____

- Find Vc(∞) _____72______

- Find   ___R/2L = 37500______

- Find wo __1/sqrt(LC) = 62500________

- Critically Damped, Underdamped or Overdamped ? _________UNDER_________________

- Find  dVc/dt(0+) ___________________________

dVc(0+)/dt = ic(0+)/C     =  IL(0+)/C  = IL(0-)/C  = .015A / 4x10^-9  =   3,750,000 V/s
Vc(0+) = 0

- Find the complete solution for v(t) for t>0

Wd = sqrt(wo^2 – a^2) = 50000

V(0+) = 72 + B1

dVc/dt(0+) =  - a B1 + wd B2

B1 =   -72

3.75x10^6 =  -37500B1 + 50000B2  ,  B2 = 21

V(t) = + 72 +  e-37500t (- 72cos(50000t) + 21 sin(50000t))