ENGR 12				Assignment 14			 Due: next wed

Part I.  Drills  -- 1 point each

1) A series RC lowpass filter requires a cutoff frequency of 8 kHz. Use R = 10 k and compute the value of C required.

2) A series RL highpass filter with a cutoff frequency of 2 kHz is needed. Using R = 5 k, compute  
 	a) L,           b) |H(w)| at 200 Hz,                    c) Phase(H(w)) at 200 Hz

3) A series RLC has a 0.1 uF capacitor. Find R and L for a bandpass filter with a center frequency of 12 kHz and a Q of 6

4)

Part II.  Assisted Problem Solving – 2 pts

| 6) Find the transfer function Vout/Vin and the corner frequency w. What kind of filter is it? Plot the magnitude and phase of H(w) using Freemat or WolframAlpha. | Plan - It tends to be easier to derive transfer functions when the component values are defined as symbols: use R1, L and R2 for the three values. - Use the voltage divider formula to derive H(w) - Then find H(w) by taking Vout/Vin - Substitute in the numeric component values, and - Plot the magnitude and phase of H(w) using Freemat. |
| --- | --- |
| 7) Find the transfer function Vo/Vs for the following circuit. Show that it is a low pass filter. | PLAN - Use L, R, and C as the component values while you derive H(w) - Simplify the formula into a simple fraction form. (remove fractions in the numerator and denominator) - Determine H(0) and H(infinity) and use this to argue that kind of filter it is |

Part III. Unassisted Problem Solving – 3 points

8)  Find the transfer function for this circuit given R1=R2=110 k, C1 = 4uF, C2 = 2 uF. Determine the cutoff frequency and plot the magnitude in Freemat.  Hint: Use Nodal Analysis at the R1/R2/C1 node and Node 3. Negative feedback means V2 = V3.